THRUSTMASTER COUGAR

CCP and TM Software manual

David Block and Mark Mooney

Version 2.7

5th July,  2004

1.1  Introduction to the CCP and TM Software manual	4

2. The CCP – Cougar Control Panel	5

2.1 The driverless controller	5

2.1.1 A warning about Windows calibration	5

2.2 Using the CCP	5

2.2.1 Default	6

2.2.2 Save	6

2.2.3 Load	6

2.2.4 Delete	7

2.3 Joystick Modes	8

2.3.1 Axis Response	8

2.4 Axis Parameters Section	9

2.4.1 Axis Parameter Buttons	9

2.4.2 Axis Setup Tab	10

2.5 Axis Shaping Tab	16

2.5.1 Dead Zone Information	17

2.5.2 Calibration Center	19

2.5.3 Axis Trim	19

2.5.4 Curve Setting	21

2.6 Startup & Calibration Tab	23

2.6.1 Startup Options	23

2.6.2 Calibration	24

2.6.3 Manual Calibration	24

2.7.1 Restart Device	27

2.7.2 Button & Axis emulation	27

2.7.3 Download to device	27

2.7.4 Poll device	29

2.7.5 Hide / Taskbar Icon functionality	29

3. The Compiler	30

3.1 Compiler function	30

3.1.1 A typical download	30

4. The Firmware	32

4.1 What is the firmware?	32

4.2 How to flash upgrade the firmware	32

4.2.1 Some definitions for you	33

4.2.2 Flash upgrading from Foxy	33

4.2.4 Troubleshooting flash problems	36

5. HOTASCleanse	38

5.1 Overview of HOTASCleanse.exe	38

5.1.1 Deleting Windows Calibration data	38

5.1.2 Erasing the Cougar’s firmware	39

5.1.3 Removing the polling registry entry	39

6. HOTASView	40

6.1 Overview of HOTASView.exe	40

7. Multiple devices	41

7.1 MultipleDevices.exe for those who want to use more than one Cougar on the same computer.	41

8. THR_RANGE acquirer	42

8.1 Overview	42

9. Button/Logical Flag Viewer	43

9.1 Overview	43

9.1.1 Button/Logical Flag Viewer Rules	43

## 1.1  Introduction to the CCP and TM Software manual

What this manual aims to cover

In case you hadn’t realised, the Cougar comes with two sets of software. The first set was developed principally by David Block with Mark Mooney responsible for the firmware, whilst the second consists of Foxy and its supporting applications developed by James Hallows. The latter set are covered in their own manual.

The aim of this manual therefore is to provide a complete reference resource on the following software:

- Cougar Control Panel – HOTASConfig.exe

- Compiler – HOTASCompiler.exe

- Firmware and loader – HOTASUpdate.exe

- HOTASCleanse - HOTASCleanse.exe

- HOTASView - HOTASView.exe

- Multiple Devices - MultiDevices.exe

- THR_RANGE acquirer - ThrottleRange.exe

- Button/Logical Flag viewer – CougarBState.exe

These applications are all installed in the same folder, along with other important Cougar software generated files, usually: C:\Program Files\HOTAS.

# 2. The CCP – Cougar Control Panel

## 2.1 The driverless controller

The HOTAS Cougar is a virtually driverless joystick solution, that does not require any programs to run in the background to apply different axis settings, or to perform emulation of any sort.

### 2.1.1 A warning about Windows calibration

For this reason, it is important that the joystick itself is aware of all changes being made to the axes, and therefore, other applications such as CTFJ (Bob Church’s Centering Tool For Joysticks), and the Windows Calibration routine should not be used - if they are, many advanced changes to the axes’ calibration will produce varied results.

PLEASE NOTE: DO NOT USE WINDOWS CALIBRATION TO CALIBRATE THE HOTAS COUGAR. INSTEAD USE THE HOTAS COUGAR CONTROL PANEL TO MANUALLY CALIBRATE THE HOTAS COUGAR.

## 2.2 Using the CCP

The HOTAS Cougar Control Panel (CCP) application is used to change various parameters for the output of the HOTAS Cougar, ranging from the mapping of axes to the different output modes of the Joystick (emulation mode, Windows mode… ).  What follows in this manual is a button-by-button explanation of the application, and how you can expect changes applied to these buttons to be reflected in the operation of the Joystick.

To display the settings that are currently active in the Joystick, open the HOTAS CCP while the Joystick is connected.  Above the help button, the current firmware version of the Cougar is displayed, note that the firmware version will be displayed only if you are using firmware version 3.06 or later.

Figure 1: HOTAS Cougar Control Panel in the default configuration Axis Profiles

Profiles are used to quick-load previously defined configurations into the HOTAS CCP and Joystick.  In the Profiles section of the HOTAS CCP, four buttons are available: “Default”, “Load”, “Save” and “Delete”, all of which are explained in the sections below.

### 2.2.1 Default

Clicking the Default button displays the Default parameters which are used by the Joystick in Windows mode.  This includes all axes mapped according to DirectX standards, most axes in the positive defined direction, upper and lower dead zones of 5%, centre dead zones of 7%, linear curve settings (0), base of curve centred, and the “apply enable/disable Windows axes states” disabled.  If the Joystick is connected and the screen display does not make any sense, the best course of action is to click on the Default button, and perform the changes from there.

### 2.2.2 Save

Clicking on the Save button saves the current configuration, which has been defined in the Axes Parameters section, to the HOTAS/Profiles folder, with the file extension ’.TMC’ (Thrustmaster Configuration).  If any Calibration has been performed, it will also be saved in this file.  Saving profiles this way means that different axes parameter preferences can be saved for different games, and can be loaded for future use.

### 2.2.3 Load

Clicking the Load button loads the saved configuration profile into the graphical display (the tabs).  This does not however load the file into the Joystick; performing this action still requires you to click on the Apply button, described further on in this reference book.

### 2.2.4 Delete

This button is used to delete a profile.

## 2.3 Joystick Modes

The HOTAS CCP allows you to set the Joystick modes pertaining to the Axis Parameters, Calibration Options, and Emulation features.  Setting these Joystick modes does not require you to click on the Apply button, as any change in the Axis response mode will automatically instruct the Joystick to change modes.

### 2.3.1 Axis Response

In Default mode, the Joystick will use the default axis setup and shaping data. This data is always loaded into the Joystick, and is used whenever the Joystick is connected - with the exception of the numbered axis buttons, should the last download have specified to enable the visible status (this is further explained in the “Changing Windows Axes States” section).  When the Axis Response feature is set to User Defined Mode, the Joystick will use the following data, as you specified:

Axis Mapping

Reversing Data

Locking Data

Curve Information

Base of Curve

Dead Zone Information

Trim Settings

For information on the Calibration options, please see the section on Calibration.  For information on the Emulation modes, please see the section on Button & Axis emulation.

## 2.4 Axis Parameters Section

The HOTAS CCP application features three tabs: the Axis Setup Tab, the Axis Shaping tab and the Startup & Calibration tab.  Performing any changes to either of these three tabs will not cause the associated action to be performed immediately by the Joystick; indeed, any changed parameters must first be applied to the Joystick, and the Axis Response must be set to User mode before the Joystick will use this new information.  To better follow with the explanations given, first open the HOTAS CCP, and click on the Default profile button.

### 2.4.1 Axis Parameter Buttons

The Axis Parameter section (excluding the aforementioned tabs) contains two buttons: “Retrieve” and “Apply”.  Their functionality and uses are explained in the following sections.

#### Apply Button

The Apply button allows you to download the configuration parameters described in the Axis Parameters section to the Joystick.  Until you actually perform this operation, the Joystick has no knowledge of any changes applied in the Axis Parameters tabs, and the Apply button will download whatever parameters are currently displayed or defined in the tabs.  For the Joystick to be able use this downloaded information, the Axes Response must be set to User Mode, which is performed automatically by clicking on the Apply button.

#### Retrieve Button

The Retrieve Button is used to upload the configuration currently saved in the Joystick.  If any of the parameters are changed in Emulation mode, these can be reflected by performing a Retrieve operation, and by checking the appropriate section.  The HOTAS CCP application automatically performs a Retrieve operation on startup, to display the Joystick’s current configuration; if the Joystick is not connected, an error message is displayed, and the Default profile is shown.  By clicking on the Retrieve button, you can also view the Joystick’s current mode.

### 2.4.2 Axis Setup Tab

Several functions are available on the Axis Setup Tab, and their respective purposes can be summarised as follows:

- Changing Axis Setup

- Reversing the direction of an axis

- Locking the values of a particular axis

- Changing the axes that are recognised by Windows

These functions are explained in detail in the respective following sections.

#### Changing the Axis Setup

If you need a particular axis directly to control a different axis in a specific configuration, the easiest way to accomplish this will be to change the axis’ setup.  When loaded in its default format with only the TQS (Throttle) connected, the axis setup will appear as such:

Figure 2: Axis Setup Tab in Default configuration

The numerous buttons represented in the centre of the Axis Mapping tab show the currently selected mapping configuration; this shows that the X axis is mapped as the first axis, the Y axis as the second, the Throttle axis as the third, etc… If we want the second axis (the DirectX Y axis) to be controlled by the throttle (which is often quite handy for racing games), we only need to click on the Throttle’s row in the second column, and the window will change appearance to look like this:

Figure 3: Axis Setup Tab with the Throttle knob as the Y Axis

As can be seen, the Throttle axis has been assigned to the second position, and the Y axis has taken the Throttle axis's place as the third axis.  All axis swapping operations can be performed this way, and although windows will “see” the axes in their changed state, all axis programming inside the joystick - including all reverse, curve, dead zone, and emulation programming - will remain specific to the physical axis; therefore, if the Throttle is programmed to have a precise curve, and is then swapped with the Y axis, the Throttle will control the Y axis in exactly the same way as it controlled the Throttle axis.

Notice that the background of the checked buttons in the sixth, seventh and eighth column are grayed, as opposed to the others (which are light blue).

This is because in this configuration the RCS (rudder and toe brakes) are not connected. The buttons with the light blue background indicate the axes which are physically connected.  To have the X and Y axes controlled by the Microstick, the Axis Setup tab would look like this:

Figure 4: Axis Setup Tab with the Microstick controlling the X & Y Axes

#### Reversing The Direction of an Axis

To reverse the direction of an axis, click on the desired button with a plus sign (or minus sign) and it will toggle the direction of the axis.  In the case where you would like to change the direction of the Y axis, the appearance of the Axis Setup tab would be as follows.

Figure 5: Axis Setup Tab with Y Axis reversed

#### Locking an Axis

On the right side of each axis name is a lock-shaped icon.  If this “lock” is open and is green in colour, then the axis is “unlocked” and will behave normally.  If the lock is closed and red in colour, then the corresponding axis is “locked”, and its value cannot be changed. To toggle the state of the lock, click on it, and it will switch between the two states described above.

#### Changing Windows Axes States

This title may not accurately reflect this section’s utility, but without getting too technical, it’s still the best description of the following section’s function.  Notice that a button is located at the top of each column, and in Default mode, buttons 9 and 10 have a red not-circle in front of the numerical text – these buttons are depicted below:

Figure 6: The "numbered axis buttons" located on Axis Setup Tab

Each of these buttons represents a different DirectX axis, these axes being:

| Axis Number | DirectX Axis Name |
| --- | --- |
| 1 | X Axis |
| 2 | Y Axis |
| 3 | Z Axis |
| 4 | Rotational X Axis |
| 5 | Slider 0 |
| 6 | Rotational Z Axis |
| 7 | Slider 1 |
| 8 | Rotational Y Axis |
| 9 | <none available> |
| 10 | <none available> |

Table 1: Axis Numbers to DirectX Names

In addition to the fact that you cannot enable the ninth and tenth axes, it is also impossible to disable the first and second axes, since the Joystick drivers’ programming implies that a Joystick features at least these two axes.  If you click on any of the buttons numbered 3 through 8, the red not-circle will be toggled on and off.  When the Joystick is connected, the names of the axes, as well as the number of axes as “seen” by Windows, are determined by one of two possible sources:

The actual physical setup of the Joystick

The states of these “numbered buttons”

Explanations of these two points follow:

#### Axes Under Physical Setup

What do we mean by the "physical setup" of the joystick? All this means is what other controllers (e.g. throttle, rudders) are connected to the joystick, which in turn defines how many axes Windows detects the Cougar as having. 6 physical setups are available for the HOTAS.

Joystick is plugged in alone

Joystick is plugged in with the TQS attached

Joystick is plugged in with the regular 1 axis RCS attached

Joystick is plugged in with the new 3 axis RCS attached

Joystick is plugged in with the TQS and regular RCS attached

Joystick is plugged in with the TQS and new RCS attached

Each of these possibilities will display a different combination of axes for the HOTAS inside of Windows.  Below is a table representing all 6 of the configuration possibilities listed above, and displaying which axes will be present for each.

|  |  | AXIS NAMES |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X | Y | Z | Rx | SL0 | Rz | SL1 | Ry |
| Configuration |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |
|  | 6 |  |  |  |  |  |  |  |  |

#### Axes Under "Enable Windows Axis States" Setup

Using the numbered buttons, you can change the axes which will be recognized by Windows.  The procedure to enable use of these buttons is listed below.

Select the desired axes which you would like Windows to “see” by toggling the state of the individual numbered buttons (i.e. enabling or disabling them), until the required configuration is reached.

Make sure that the “Apply enable/disable Windows axes states” check box is selected.

Load the file into the Joystick by using the “Apply” button, and click ‘OK’ to perform a restart of the Joystick.

Now, the Joystick will be “seen” by Windows, featuring the axes you specified.

The point of this is that even if you do not own Rudders, or the new Rudders set (with toe brakes), we can make DirectX “believe” that either of these devices are present, and then control their input values with any one of the available axes, or even through an emulation file.

It is important to note here that the Joystick will load in the user-defined “Enable Windows Axes States” mode as long as the last file downloaded to the Joystick had the “Apply enable/disable Windows axes states” check box activated.

## 2.5 Axis Shaping Tab

The Axis Shaping Tab contains controls to modify more advanced Joystick configuration features, and is very useful when it comes to enabling users to tune each axis according to their own personal preferences.  The “Axis to set” combo box at the top of the tab selects the desired axis, among the ten possible axes, whose properties you want to adjust.  Once a parameter has been changed, the associated change is graphically displayed on the graph located to the right of the list of different parameters.  The change in display is triggered by either switching parameter categories, or by clicking on the Refresh button located to the top right of the tab.  The various adjustable parameters are as follows:

Upper Dead Zone (UDZ)

Lower Dead Zone (LDZ)

Center Dead Zone (CDZ)

Calibration Center

Axis Trim

Curve Settings (factor and base)

The parameters listed here can be enabled by selecting different Joystick output modes.

### 2.5.1 Dead Zone Information

By changing the Dead Zone Information (UDZ is Upper Dead Zone, LDZ is Lower Dead Zone, CDZ is Center Dead Zone), you can alter the inactive regions on each axis.  The effects of successive changes will be reflected in the graph to the right-hand side of the area where the parameters are listed, dead-zone areas being highlighted in red.  All values have a maximum of 100%, where 100% is approximately 30% of the actual axis movement on the Joystick.  For instance, the default value used internally by the Joystick is 1% of each of the dead-zone areas, which gives a total of 3% of the complete axis travel.  Below is a pictorial representation of this tab:

Figure 7: Axis Shaping Tab with Default Parameters

In the above figure, small red regions are apparent to either side as well as at the center.  This plot shows the relationship between the physical position of the joystick axes, and the actual values that will be provided to Windows, for use in games and on your desktop.  For reference, the horizontal axis of this graph represents the values that the Joystick collects from its axes, with the minimum value displayed on the left-hand side, in this plot.  The vertical axis of the graph is

the actual output from the Joystick as it will be “seen” by Windows, with the minimum value displayed at the bottom, in this plot.  Notice that in the dead-zone regions, the graph becomes a straight line, meaning that the output will be the same for various inputs.  The lower dead zone is located on the left-hand side of the graph, and is a horizontal line; the upper dead zone is positioned on the right-hand side of the graph, and is also a horizontal line.  The next figure illustrates what happens if we increase the CDZ value:

Figure 8: Axis Shaping Tab with Y axis CDZ increased

Figure 8 clearly shows that raising the Center Dead Zone value from 7% to 25% has proportionately increased the red highlighted region at the center of the horizontal axis, and the white horizontal line at the center represents the region of the axis where the Joystick will be inactive.  The upper and lower dead zones react similarly.

### 2.5.2 Calibration Center

This value is the position that the Joystick axis will consider to be its central position.  If the specified Center Position value is lower than the Joystick’s current physical central position, then the axis will appear to reach a higher value when the Joystick is at rest in its central position.  A similar result could also be attained by adjusting the Trim value in order to offset the physical values that the Joystick outputs; the Trim function’s main advantage over the Center position feature is that Trim can be altered during emulation.

### 2.5.3 Axis Trim

The Trim function is used to offset the actual physical value of the axis to a new value.  For example, if the Joystick axis is at a physical position equaling 30%, whereas Trim is set to –20%, then the actual position understood by the computer will be 10%.  The maximum Trim setting is +/- 50%; this means that from the center position, it is possible to have the axis go from one end to another by setting the Trim.  This is demonstrated in the figures to follow.

Figure 9: Trim setting at maximum

With Trim applied to the Y axis, the reaction can be observed immediately.  With the axis at it’s physical center (the horizontal center), the output data that Windows receives is the maximum Y axis value.  If the Y axis is moved in a direction increasing its value, there will be no reaction; if the Y axis is moved away from its physical central position down to its minimum position, the computer will observe that the axis’ values are decreasing, until they reach a minimal output value, equal to the axis’ normal center position.

If we were to reduce Trim to its minimum value, the graph would look as follows:

Figure 10: Trim Setting at minimum

With this change applied, the Y axis value will be read as being minimal when the Y axis is physically at its central position, and will be read as being at its center when the Y axis physically is maintained at its maximum position.

### 2.5.4 Curve Setting

This setting can be adjusted to have the axis respond with a more exponential feeling, i.e. a “curved” feel, instead of a regular linear feel.  The Curve parameter can have values between -32 and 32; however, setting several axes to such high values will significantly slow down the operation of the Joystick.  Notice that values greater than 20, or smaller than -20 are not useful, and that the change in the curve beyond such values is not sufficient to even justify using them.

We will first take a look at curves with a centered base as is the norm for joystick axes for example. So what might be the difference between a positive curve and a negative curve?  Let’s look at the response when curve settings of positive and negative 10 are applied.

The figure to the left represents the axis with a Curve setting of -10 (negative); as can be seen, around the central position, very little change in the axis - almost like increasing the central dead zone. The rate of change then increases quickly, until the axis’ slope becomes almost vertical.  This means that the Joystick will offer enhanced responsiveness around the limits of this axis’ travel.  The figure to the right shows a graph of the axis with the Curve setting at 10, which appears to almost be the previous setting reversed; the axis is much more responsive around the physical central position, whereas at the extremities of the axis travel, responsiveness is far lower - almost like increasing the upper and lower dead zones.

The idea behind the ‘Base of Curve’ radio button is that although the response curves of the Joystick and Microstick axes should be curved, as we saw in the previous example, a full range ramp may however be more appropriate for axes such as the toe brakes and throttle.  Below are the graphs for Curve settings of positive and negative five (as a value, ten is excessive), but with the Base of curve setting set to Zero.

The figure to the left represents the graph for an axis with a negative 5 Curve setting - notice how similar it is to the upper right section of the graph representing the axis with a Curve setting of negative 10 and a centered Base of curve. The Joystick will now react very slowly at the lower extreme of the axis, as if an increased lower dead zone had been added to the axis, and its responsiveness then increases until it reaches a peak at the upper limit of the axis travel.  To the right is a Curve with a setting of positive 5, its base being Zero-based. Once again, notice how similar the entire graph for the Zero base positive curve is to the upper right section of the Center-based positive curve. Responsiveness increases near the physical minimum of the axis, and decreases around the maximum.  The upper limit of the axis looks as though we have an increased upper dead zone.

## 2.6 Startup & Calibration Tab

The startup and calibration tab contains information about how the Joystick will behave when the computer starts up and calibration configuration.  This tab is separated into two sections, the top part is the startup options and the bottom part is the calibration configuration.  Each of these sections is described below.

Figure 11: Startup & Calibration Tab

### 2.6.1 Startup Options

There are three options that the Joystick can be set to automatically when the computer starts up.  These options are the Axis response, the Calibration mode and to load a specific profile.  The Joystick, by default, will load in default axis response mode, auto calibrate mode and with the last profile that was downloaded to the Joystick.  To change the default, click on the desired options and click on the ‘Save’ button.  To choose a profile, click on the checkbox to load a profile and then click on the ‘…’ button or type the profile name directly in the edit box to the right of the ‘…’ button.  Please note that the profile must exist in the profiles subdirectory off the HOTAS directory.  To set the startup options back to the default, click on the ‘Default’ button.  It is worth noting that the Joystick can never be placed into Manual Calibration and Default Axis Response, this is why when Manual Calibration is selected, even in the Startup Options, the Axis Response is automatically set to User Mode.

### 2.6.2 Calibration

The Calibration section contains options to set the calibration mode, calibrate the Joystick, and select the active calibration file to be used when downloading a profile to the Joystick.

Choosing Auto or Manual calibration in the Calibration Mode option will automatically switch the Joystick to the desired option.  Please note that it is not required to click on the ‘Apply’ button to apply the calibration mode – this happens immediately.

In Manual calibration mode, the data used will be that which was loaded into the Joystick during a profile download, using the currently selected active calibration file.  This data is automatically downloaded, along with the current axis parameters on any “Apply” of profile data.

Auto calibration mode means that the Joystick will configure the axes to their maximum positions as you move the axes to the extremes.  Manual calibration means that the Joystick will use the calibration data that was created by performing the manual calibration routine, described later in this section.  Please note that the Joystick can not switch to Manual calibration mode when the Calibration data contained within the Joystick is not representative of the hardware connected.

If the Joystick is reset (by reconnecting it, or clicking on the Restart button) and is in Auto-calibrate mode, it is recommended that you move all the Cougar axes (joystick, throttle, Range, Antenna, Microstick etc) to their maximum and minimum positions, holding them at these positions for about 3 seconds. This will allow the auto calibration to gather information from the axes and accurately calibrate your controller axes.

### 2.6.3 Manual Calibration

The Manual Calibration button will bring up the Calibration Routine window, shown below:

Figure 12: Manual Calibration Routine Window

Follow the directions given, and click on the Next button when the axis has reached the position that is to be saved for that particular step.  For instance, if in the second state you move the Y axis to its forward position and then click the Next button, the Joystick will save the position that the Y axis was in at the time when you clicked the button. When you are done with the calibration setup, the application will automatically send the calibration information to the Joystick, along with all the data currently loaded into the axis parameter section.  This calibration data is saved, and can be saved to a profile for later use.

Note that the axes that are not physically connected are grayed out.  The calibration routine will skip over the axes that are not connected and not record their values for use in the calibration file.  The progress bar at the bottom left hand side of the Window indicates the ‘raw’ values of the axis – it may not be possible to reach the maximum or minimum positions of the progress bar.  It is to be used as a guide to show the correct axis to be moved and the correct direction.

Upon completion of the manual calibration sequence the Save Calibration Data window will open asking for the name of the file in which to save the data from the calibration just performed.  The window also presents the options for setting this calibration as the newly active calibration file, as well as to apply this calibration data now along with the axis settings currently showing in the CCP and switch the Joystick into Manual Calibration Mode.

Figure 13: Save Calibration Data Window

If, for example, the last profile was download while the active calibration file contained only data for the Joystick and Throttle axes, then the next time the setup was connected, rudders were added, the CCP will automatically disable the ability to switch to Manual Calibration mode.  Should the currently selected active calibration file’s data not match the hardware currently connected the CCP will display a message explaining this below the drop-down box for selecting an active calibration file.  The figure below represents both of these cases.

Figure 14: Calibration section while downloaded and selected calibration data is incorrect.

There is a separate Calibration manual installed with Foxy. Use that to understand everything there is to know about calibration.

2.7 Actions and other options

In the Actions section there are three buttons: Restart Device, Button & Axis emulation and Download to device.  There are also options for the automatic polling of the Joystick and the Hide button.  All these actions and options are described below.

### 2.7.1 Restart Device

The Restart Device button will cause the Joystick to perform a manual ‘unplug’, this is similar to disconnecting and reconnecting the Joystick from the USB port.  This function is useful whenever you wish to change the Windows Axes States.  In this case, it would be necessary to disconnect and reconnect the Joystick (as explained in the section “Changing Windows Axes States”), which is effectively accomplished by clicking on the Restart Device Button.  Also, on startup, the Auto Calibration Routine measures the centre positions of the relevant axes (X, Y, Rudder, and microstick axes); to manually set the centers of the axes, click on the Restart button, and hold the axes at their desired positions.

### 2.7.2 Button & Axis emulation

If the Button & Axis emulation is ON (green background), then the Joystick will use the last emulation file which was downloaded to the Joystick.  The emulation file is the file that controls the keyboard and mouse emulation, as well as the various axis parameter changing features. Please refer to the Cougar Owner Reference Book for further information.

If the Button & Axis emulation is OFF (red background), then the Joystick will behave like a simple Joystick with buttons activating DirectX buttons in Windows.

### 2.7.3 Download to device

The Download to device button opens the HOTAS Cougar Loader application.  The Loader application is used to download joystick files (Thrustmaster Joystick files .TMJ).  For more information about the structure of an emulation file, please see the document “HOTAS Cougar Owner Reference Book.”  On the next page is a picture of the HOTAS Cougar Loader application.

There are two buttons and a checkbox on the main HOTAS Cougar Loader window.  The Check button is to check the joystick file for errors.  This will not download the file to the Joystick.  The Download button is to check the joystick file and then download it to the Joystick if there are no errors.  The section in

Figure 15: HOTAS Cougar Loader to download joystick files

‘Currently loaded file properties’ shows you which joystick file is currently loaded in the Joystick and the date the last download was performed.  The checkbox for ‘Turn on emulation mode after downloading’ is used for turning the emulation mode on after a joystick file has been downloaded to the Joystick.  Once a joystick file has been downloaded, the emulation mode must be turned on for the Joystick to emulate the keypresses and axis movements stored in the joystick file.  This can also be done by clicking the Button & Axis emulation button in the HOTAS CCP.  See the section ‘Button & Axis emulation’ in the last section.  When checking or downloading the joystick file, the following window will appear.

Figure 16: HOTAS Cougar Compiler to check joystick files

### 2.7.4 Poll device

The checkbox for polling the device is used for the HOTAS CCP to poll the device for its current status.  Polling will check whether the Joystick is connected and which mode the Joystick is in.  The interval time for the polling is set in the edit box underneath the checkbox.

### 2.7.5 Hide / Taskbar Icon functionality

The HOTAS CCP can be hidden with only the icon showing in the taskbar.  Click on the ‘Hide’ button to hide the HOTAS CCP.  Note that this functionality can only be used with the polling turned on.  To show the HOTAS CCP after the application has been hidden, click on the taskbar icon with the left button and choose ‘Open HOTAS Cougar Control Panel…’ from the menu that appears.  The other options available from the menu are to exit the HOTAS CCP and to open the HOTAS Cougar Loader.  You can switch the Joystick to and from emulation mode by clicking on the taskbar icon with the right button.  The colors of the icon will change according to the following table:

| Icon | Colour Code | Description |
| --- | --- | --- |
|  | Green fighter / Yellow BG | Emulation mode is ON, Axis response is in User mode |
|  | Red fighter / Yellow BG | Emulation mode is OFF, Axis response is in User mode |
|  | Green fighter / Gray BG | Emulation mode is ON, Axis response is in Windows mode |
|  | Red fighter / Gray BG | Emulation mode is OFF, Axis response is in Windows mode |

Table 2: Taskbar icon descriptions

# 3. The Compiler

## 3.1 Compiler function

The compiler checks the joystick and macro files for errors, converts them into a format that can be used by the firmware inside the Cougar (ie. it compiles them), and downloads the compiled data to the Cougar joystick.

The compiler is the unsung hero of the Cougar software. It an application that you’ll never run on its own with no graphical front end, as you’ll do all your compiling and downloading of files from Foxy or one of its applications. This is an important point you will see me stress throughout the software manuals. Foxy cannot talk to the Cougar directly. Whenever it wants the Cougar to do anything, like downloading a file, changing modes, etc. it sends a request to the TM software and asks it to do its hard work for it.

The compiler in my mind is the most complex of the Cougar software. With all the permutations that can be encountered in a joystick file, it is the compiler’s job to make sense of that and ensure it can be converted into a format the firmware within the Cougar can use. If there’s an error in a joystick file, the compiler will send a message back to Foxy to let it know what the error was and where it was located, and then Foxy regains control to inform you the user what the problem is.

### 3.1.1 A typical download

If you’re downloading a file from Foxy or any of its applications, and you’d like to know how the software handles a download, this next section is for you. Otherwise, move swiftly on ;)

1.) Foxy sends the joystick file name and location to the compiler.

2.) The compiler checks the file for errors. If there are errors, it sends a message back to Foxy indicating what the problem is and where in the file it is located. The download is prevented. If there are no errors then the compiler asks the CCP to determine whether a Cougar is connected and whether the communication to it is fine. The CCP will not be shown but it will run as a windowless application in the background.

3.) If the CCP returns a communication/connection issue, it will inform the compiler of this and an error is raised. Again the download would be halted at this point.

4.) If all is good then the compiled file is passed to the joystick, and the CCP is instructed to put the Cougar into programmed mode (depending on settings within Foxy) as well as asked to setup the various axes states, curves, etc.

5.) If all goes well the CCP and compiler unload and control is passed back to Foxy.

##### Notes

1.) Downloading a file that changes the number of axes reported to Windows, can take several seconds. This is normal. The Cougar being a driverless system does everything within itself, and therefore if an axis is going to be disabled, the Cougar has to send a message to Windows to disconnect itself, and then reconnect it with the new axis number and ordering. It takes time for Windows to go through that process – hence the delay.

2.) The compiler will generate error codes for different errors it encounters in a file. There is a file in Foxy’s Info folder (Error codes.rtf) that contains a description of those error codes (it’s too large to put into the help file). When an error is encountered, Foxy will look up the error number and show the relevant portion of that file.

3.) If something goes wrong during a download and the procedure seems to lock up Foxy or your computer, it is likely that the compiler and/or the CCP will have hung. You can see this if you Ctrl Alt Delete and look at what processes are running. If you seem them there, then personally I find it best to just disconnect the Cougar, and reboot, possibly also re-flashing the firmware after connecting the Cougar with the trigger held in. See the firmware section for more information on this.

# 4. The Firmware

## 4.1 What is the firmware?

The firmware is the microcode within the Cougar that determines the Cougar’s abilities, in terms of the resolution of its axes, its programmability and its calibration information.

Flashing the firmware is such a fundamental task that you must get comfortable with, as it will get you out of trouble should you run into problems with your Cougar 99 times out of 100! There’s already a section in Foxy’s manual on how to flash the firmware, so I’m just going to replicate it here. This is vital information you must take on board and you must get used to and be happy to flash the Cougar’s firmware.

## 4.2 How to flash upgrade the firmware

If you've ever flashed the BIOS on your motherboard, then you'll also probably remember the first time you did this and how petrified you were. Well I certainly remember that fear. This was because I'd read nightmare accounts on the net from people who after flashing their BIOS couldn't get their computers to start, booted up with blank screens, had to use jumpers to clear their CMOS settings .... etc. etc. I've noticed that this trepidation has trickled down to users when advised to flash their Cougars. What if it all goes horribly wrong?

So I want to set you straight on this. It won't go horribly wrong. It is likely to become such a non event for you you'll wonder what all the fuss was about. If for any reason the flash doesn't take, a reboot and reflash will always fix it. You can completely erase the firmware inside the Cougar so it's gone, and it's no big deal. The stick will quite happily sit there still connected, ready to be flashed again, and your compute will still work fine. So hopefully you can lay your fears to rest on this. During development, when we were testing statements and bug hunting, we'd always reflash before every test to ensure we were at the same common state, and that might mean we'd be flashing our controllers 30+ times a day. So it's no big deal, and it's a great way of resetting your controllers to a perfect state should you have any problem with them.

### 4.2.1 Some definitions for you

Before I proceed, I'd better explain what we mean by some terms you will see used in relation to this subject:

Firmware: This is the posh term for the "microcode", a set of instructions residing within the Cougar, that determine the Cougar's capabilities, and interprets programming and axis data held within the Cougar's memory, presenting it to the operating system and hence your games.

Flash Upgrading: Also known as "flashing," is a way of transferring the Firmware to a part of the Cougar's memory. This memory is retained irrespective of whether the Cougar is connected/powered or not.

HOTASUpdate.exe: This is the executable that when run transfers the firmware it contains to the Cougar.

Let's move on to how to upgrade your Cougar's firmware then.

### 4.2.2 Flash upgrading from Foxy

1.) Before we start, ensure that none of the Thrustmaster applications are running - like the Cougar Control Panel (CCP) or Loader. There's no problem in having Foxy or the Launcher running, even with Cougar mode detection on, but just make sure that the CCP isn't running.

2.) When it comes to doing anything with setting up the Cougar, it's all fairly straight forward as to where to go in Foxy, because there's a menu called ... "Cougar" and as luck would have it, one of its menu items say's "Flash upgrade" - so we'll select this.

3.) And when we do (if your throttle is connected), this Window pops up.

This message has been added because of reports of users having problems with slow throttle response directly after flashing the firmware.  In order to protect users unaware of the possible issue, this message warns the user to unplug their throttle before continuing the flash sequence.  If you’re content to continue with the throttle plugged, check the available checkbox, if not, follow the instructions given.

4.)  When the Update is re-run with the throttle unplugged (or continued using the checkbox) the following Window pops up.

You'll notice on this screenshot I've highlighted the firmware version we'll be flashing the Cougar with. With the current software you are able to see the version number in the CCP provided the firmware in the Cougar is more recent than 3.06.

5.) To update the firmware then just follow the instructions as they appear in bold. Note that if your Cougar is in Programmed mode, the updater will place the Cougar into Windows mode.

6.) The first instruction as we can see from the above screenshot is to hold in your trigger. Wait for a around 2 seconds, with your trigger held in and then you'll soon see the second line of instructions in bold, like this:

7.) At this stage, release the trigger and then sit back, eat some chocolate and gaze vacantly around the room. After about 4 seconds the firmware is installed and this window should pop up:

You'll see that it's recommending that we move the axes around to allow the autocalibration to read in values from the axes and configure itself for optimal response from the axes. I do recommend though that you wait about 5 seconds before touching the joystick/throttle and moving their axes around, as the Cougar is still being redetected by Windows. What I tend to do is to have Foxy or the Launcher running with their Cougar mode detection on. During the flash process and during subsequent detection of the Cougar, this will be yellow or show an error. Wait for it to turn Red and show that the Cougar is detected and in Windows mode, before moving the axes around.

That's it - your Cougar is flashed with the firmware of your choice, and you're up an running. Well, you should go and test that you're up and running, and I'll write some articles on testing the Cougar using Foxy's various apps soon.

One last point I want to emphasize is this: Reflashing the Cougar is an excellent way of ensuring that your Cougar is clean of all issues and at a stable perfect state from which to test code and get going from. So if you do run into problems with the Cougar, and let's face it - it is new technology, from a new team, so it's likely to have a few teething problems - then remember that this technique will always sort them out. I like it! (And I'm sure Technical Support do as well!) ;)

### 4.2.4 Troubleshooting flash problems

It's very rare for a flash not to take, but the commonest reason from my experience when the Cougar was being developed, was when the software couldn't communicate with the Cougar. An easy way to test this is to try and change to Programmed or Windows mode (even if there's no program loaded.) If there's a communication problem you will likely see this Window pop up:

At this stage, don't try and flash the Cougar. Try and fix the communication problem. Now there are various ways - the simplest being to unplug and then reconnect your Cougar. That will work nearly all the time. If it doesn't, it could be because one of the applications is hung (you'll usually see it if you press Ctrl Alt Delete) in which case, leave the Cougar disconnected, and reboot your machine. We could then reconnect the Cougar and flash it from there, but I'm going to show you the ultimate trick now.

Instead of just plugging in your Cougar as normal, first, hold your trigger in, and keep holding it in, as you plug your Cougar in. Once the Cougar is connected, let go of the trigger. What this does is to clear the firmware completely from the Cougar. This is a very handy trick to remember - a bit of an overkill but I tend to do this myself as old habits die hard ;). If something really does go badly wrong with your Cougar, holding the trigger in whilst you reconnect it, will erase the firmware and leave you with a clean empty stick. Now all you have to do is to flash the Cougar. Again you do this in exactly the same way as before, but this time, a Window will not pop up asking you to depress your trigger. It'll just go straight ahead and flash the firmware for you and just notify you when it's done. Handy to know that!

Here's another little piece of advice. If you're reflashing the Cougar because you've had problems flashing it before or you've had some software crashing, then when you start up the CCP for the first time after the flash, you may find that the CCP is unresponsive initially and eventually comes up with a message: "Waiting for device to become available." If this happens just wait - the CCP will pop up eventually after saying that the "device didn't become available in time" or something to that effect. Just close the CCP down at this point. To fix this issue you can do one of two things. Either download a file to your Cougar, or run the HOTASCleanse.exe (see Foxy’s Cougar menu) and select Erase polling registry entry. The reason this problem may arise is because when any of the TM apps do anything to the hardware, a flag is set in the registry that prevents any other software (HOTASCompiler, HOTASUpdate, HOTAS CCP) from communicating with the hardware. Once the software is finished it removes the flag. What happens sometimes is that if the software crashes in the middle of the transfer or something, then this flag remains set. From that point on, when the CCP tries to communicate with the HOTAS, it will display the status message "Waiting for device to become available." Just doing a reboot won't remove the registry flag. When you run the HOTASUpdate and reflash the HOTAS, it will clear that registry flag for you. That Erase polling registry entry in the HOTASCleanse.exe does in fact that. It removes the registry flag for you.

# 5. HOTASCleanse

## 5.1 Overview of HOTASCleanse.exe

HOTASCleanse was introduced with a later version of the TM software. It’s primary purpose is to delete calibration data in the registry when users use Windows calibration routines with the Cougar. Using such routines can result in a non functional Cougar, axes that show only two values, or axes with improper calibration.

This application can be run from Foxy’s Cougar menu. Currently this application can do 3 things:

- Delete your Windows Calibration data

- Erase the Cougars firmware

- Removing the polling registry entry

### 5.1.1 Deleting Windows Calibration data

The first one is probably the most useful out of all of these given the issues we’ve seen on the Cougar World forums. Let me repeat a vital message you must imprint somewhere inside: You must not use the standard Control Panel Joystick calibration routines or any third party calibration applications. These store calibration data in the registry and they can cause Cougar disconnection problems, missing axes, and axes not going full range. A classic sign of Windows calibration data effects is an axis that gives just 2 values – middle and max, and swapping axes via the CCP results in this effect staying where it is irrespective of which physical axis is controlling that axis.

There’s a section elsewhere called Calibration Do’s and Don’ts that covers why you shouldn’t be using the Windows calibration tools, but let’s get back to how to use this application correctly.

- Run HOTAS Cleanse and delete your Windows calibration data, with your Cougar connected.

- At this stage, open up the Control Panel’s hardware device manager and see if you have an HID Human Interface Compliant Game Controller. If you do, uninstall it. You probably won’t but I know one tester found this step essential so it’s definitely worth mentioning here.

- Disconnect your Cougar and hard reboot (power down totally, and then turn on your computer.)

- Reconnect your Cougar and for good measure, reflash the firmware.

- Move the axes around as directed to reinitialise the autocalibration routines.

### 5.1.2 Erasing the Cougar’s firmware

As you will hopefully have read from the firmware section covering how to flash the firmware, it is possible to remove the firmware from the Cougar by connecting it with the trigger held in, usually after a reboot if things have gone that pear shaped. But here’s another way if all you want to do is clear the firmware and don’t need to do the trigger in/connect trick.

### 5.1.3 Removing the polling registry entry

For an explanation of what this does, see section 4.2.4: Troubleshooting flash problems.

# 6. HOTASView

## 6.1 Overview of HOTASView.exe

HOTASView shows the axes values and button presses when the Cougar is in Windows mode.

In many respects it’s similar to the analysers in Foxy. It is accessible from Windows Control Panel as well as Foxy’s Cougar menu. That’s it! It’s one of those applications you just run and see what it does straight away, so no real need for a manual. Apologies therefore for wasting paper if you’re printing this out.

# 7. Multiple devices

## 7.1 MultipleDevices.exe for those who want to use more than one Cougar on the same computer.

The Cougar software can only talk to one Cougar connected to your computer, the active Cougar. It is possible with this software, and with multiple Cougars connected to one computer, to assign which of them is the active Cougar for the software to communicate to it.

As the Cougar is a USB device, it is possible for you to own and connect more than one to your computer. A problem arises though if you want to download a joystick file to more than one Cougar, or if you want to change Cougar modes, etc. How will the software or Foxy know which Cougar you want to perform some action on?

The only way therefore is to run MultipleDevices.exe so that it can assign the active Cougar. Then it’s a simple matter of setting up that Cougar as though it was the only Cougar connected to your machine.

Notes

1.) Many flight simulators are only capable of recognising one controller, so do not be surprised if you can only get one Cougar recognised in a game.

2.) Be careful to test each Cougar out to ensure that it is in the mode that you want it to be in, as well as programmed how you want it. By default the Cougar software and Foxy uses a single file, HOTAS.ini to save hardware and other information. That file isn’t going to change when you assign a different Cougar using MultipleDevices.exe, so it will be possible for Foxy to report the mode incorrectly at first until you do something to change the mode etc. This will also affect for example if you have non toebrake Simped rudders connected to one Cougar and not to the other, as with the rudder connected Cougar you normally have to change the “RCSType= xx” setting in the HOTAS.ini file. So my advice here would be to test each Cougar in one of the analysers/key tester.

3.) Don’t use the Launcher to download files and setup multiple Cougars – because it’s can’t. It will only set up the active Cougar.

4.) Windows allocates a controller ID in Gaming Options (Control Panel) to all connected controllers. You will need to experiment or ensure that the controller ID’s stay as you would expect them to, for correct use in your sims.

# 8. THR_RANGE acquirer

## 8.1 Overview

The ThrottleRange.exe is an application that can be run to collect position information from the throttle axis. This is used in programming statements that restrict the analogue range of the throttle between specific physical throttle positions.

For more information on this, see section “5.9  The THR_RANGE statement” in the Statements and Syntax manual.

# 9. Button/Logical Flag Viewer

## 9.1 Overview

The Button/Logical Flag Viewer is a graphical representation of the current button and logical flag states inside the emulation engine.  Basically what this means is that with this application open, it is possible to see which buttons or flags are on or off allowing the user to see how their joystick file is operating.

When a Joystick file is active in the Cougar, the button states which can be seen in the HOTASView application or through Foxy’s analysers are the button states which have been programmed into that file, and not the true physical state of the buttons themselves.  All the logical flag states as well, are only used internally by the Cougar, and their state can only be seen normally through what actions are associated with them in the Joystick file.  In order to view these states while the Joystick file is active (Button & Axis Emulation On), it is necessary to use the Button/Logical Flag Viewer.  This viewer shows at one time all 32 buttons and all 48 logical flags.

### 9.1.1 Button/Logical Flag Viewer Rules

There are a couple of rules to remember concerning the Button/Logical Flag Viewer.

- The logical flag viewer must be the application in focus in order to see the states of the physical buttons and logical flags.

- If the Joystick file is not active, none of the buttons or flags will light up, not even if the physical buttons are pressed.