Version 2.1

Author: James Hallows

Last modified 22nd October 05

Calibrating the Cougar

Introduction

A basic requirement of any joystick is that it should be able to be calibrated. Calibrating the Cougar is a little different to most sticks, so this document exists to show you how to use auto or manual calibration successfully.

Definition: Calibration is simply a way of ensuring that all the Cougar axes can reach the same maximum and minimum values corresponding to their physical range of travel.

It is achieved by measuring the minimum, centre and maximum values that are generated for each axis, and scaling the axis readings and presenting them in a range Windows expects to see.

Before we get going though, an essential warning is in order:

Never use any non-Thrustmaster calibration routine

Windows has its own calibration routine for joysticks in its Game Controllers applet and you must not use it with the Cougar. It and others will create and store calibration data in the Windows registry, and this will adversely affect the Cougar’s calibration. Please see the documentation in the installed (not printed) CCP manual on the HOTASCleanse.exe application should you think that you have calibration data in the registry affecting the Cougar. Don't just run the application – it won't clear your registry calibration data properly. Follow the instructions in the installed manual.

Second warning: I have the ability to write an awful lot to explain the simplest of procedures, so don’t be put off by the length of this document. It’s a gift. There’s a summary at the end but I do recommend that you keep a copy of this in your downstairs loo and read through it until it all makes sense.

One final point: This document assumes that you are using the latest TM software and firmware. There have been significant changes made to the calibration routines since the software that originally came with the Cougar. Make sure you're running the latest version of everything, including updating your firmware.

Off we go then.

Two methods of calibrating the Cougar

Here’s a quick overview before we get our hands dirty. As the title says there are essentially two different ways to effect a calibration of the Cougar. They are by:

1.) Auto calibration.

2.) Manual calibration.

Unlike most joysticks that store calibration data in Windows, the Cougar uses only the calibration data inside its memory. It actually has two areas of memory it can use for calibration data – one for auto calibration data, and the other for manual calibration data. When you select the Calibration mode on the CCP’s Startup & Calibration tab, you are effectively switching between these two areas of memory.

Auto calibration is the default setup and is the easiest to understand and use. Calibration data is re-calculated continuously whenever the firmware reads different extremes of values from each axis. This data is cleared and effectively re-initialised whenever power is lost to the Cougar, as well as in a few other situations.

Manual calibration relies on calibration data stored inside the Cougar. This data gets into the Cougar from a Cougar Calibration File (*.ccf), a file saved on your computer containing calibration data created when you run through the CCP's manual calibration routine.

When you’re messing around in the CCP calibrating or changing axis

data, I personally recommend that you ensure that the stick is in Windows

mode  (Button and Axis Emulation mode is off.)

Let’s get our hands dirty then …..

1.) Auto calibration

The simplest and default way of calibrating your Cougar. Open up the CCP and make sure first that the Calibration Mode is set to Auto calibration and then secondly that the Axis Response is set to Default mode.

Now all you need to do is to move all axes to their maximum positions, holding them there for a few seconds, and then to their minimum positions and holding them there for a few seconds. The Cougar will collect all the data it needs for calibration and you are done. Go flying. The Cougar will continue to update its calibration information as you use it.

Auto calibration can be used with the Axis Response set to Default mode or User mode. Default mode is the default axis setup and ordering. User mode reflects any modifications to the axis settings you’ve applied. Very commonly, the CCP is used to modify axis settings (see the CCP manual) and then the settings are sent to the Cougar when you press the Apply button. This will apply the axis settings to the axis data in the stick and automatically switch the Axis Response to User mode.

You can affect the axes through axis programming statements in your joystick file even if the CCP says Default mode. Just something to be aware of.

You can if you’re running at a high enough resolution, run Foxy’s Windows joystick analyser besides the CCP at this point, so you can monitor the effects of your changes. This is great for checking calibration and axis information as you adjust it.

Notes

- Auto calibration data is lost every time your Cougar loses power, on firmware updates, and when switching between Default and User mode. This is by design.

- If when switching from User to Default mode, you find that the axes aren’t reaching maximum and minimum positions in Default mode, then switch back to User mode and then straight back to Default mode again. This can occur if you’ve used auto calibration with deadzones removed from the axes.

- If you start removing deadzones, or fly frantically online when you really do throw the stick forcefully around, it is possible to generate auto calibration data that is difficult to go full range with normal stick movement. If you’re in doubt, open up Foxy’s joystick analyser and see what your stick’s response is up to. If you do find this, then re-initialise the auto calibration data, or better, use manual calibration.

- The Cougar will always default to auto calibration after updating the firmware.

- You can instruct the CCP to place the Cougar in manual or auto calibration mode when Windows starts by several means. You can set the Startup Options from the CCP’s Startup & Calibration tab, or you could use the Launcher to automatically download a file containing a USE PROFILE statement with suitable syntax to switch the calibration mode.

2.) Manual calibration

In this section I’ll explain how to perform a manual calibration, how to store that data, and how to use manual calibration reliably. Before we get into the details, let's keep at the back of our minds an overview of how we achieve a manual calibration:

Overview

1.) We'll run through the manual calibration routine, collecting values generated from each axis.

2.) We'll store that information in a Cougar Calibration File (*.ccf), using the default file name, calibration.ccf.

3.) We'll then instruct the CCP that it should always use this file as its source of manual calibration data. We do this by telling the CCP that we want the calibration.ccf file to be the Active Calibration File.

4.) We instruct the CCP at the end of the manual calibration routine to place the Cougar into manual calibration mode using the data we just gathered.

Performing a manual calibration

Right – nice and simple:

1.) Ensure your Cougar in Windows mode (big red button should be visible). You may want to ensure that the Cougar is in auto calibration mode and that you’ve moved your axes around. In theory it shouldn’t make any difference because the calibration routine looks at raw axis data from the stick, and not what is presented to Windows. However I have seen people posting on the forums (usually those with hall sensors) saying that setting the centre positions helped them overcome manual calibration error messages. Not something I’ve encountered.

2.) Press the Manual Calibration button.

Don't worry if in your CCP, in the Calibration Mode frame, the manual calibration option is disabled. It will be enabled after a successful manual calibration is performed.

3.) The calibration routine window pops up. Make sure all your axes are centred as requested and then run through each stage of the calibration routine. You should look at the Raw values area down the bottom left to judge when you’ve moved the axes to their maximum or minimum values.

On the screen capture below I’ve indicated which axis directions result in the lowest value being displayed in the Raw values area, and therefore for all the other axis directions, you’re after the highest values. In fact to be honest it doesn’t matter too much which way round you do this, as the CCP will sort it out if you do it the wrong way.

Anyway it’s a simple matter of rattling through this all and getting to step 20.

The CCP should correctly detect the number of axes present, and disable those not present when running through the calibration routine. So the above screen capture represents a Cougar with rudders attached that don't have toebrakes.

A common question asked is “which way is clockwise when rotating the ANT and RNG rotaries?” By convention, looking at the knob from a “face on” direction dictates the rotation direction.

At stage 20, hit the Apply button. This window pops up:

As the window title says, we're now being asked about saving the calibration data we just collected. Let's get into a little more detail on this.

Calibration data is stored in Cougar Calibration Files (*.ccf) in the Calib folder in your main HOTAS folder. You can create as many calibration files as you want to, which can be useful for example if you have Cougars with different mods in them, or want to create calibration data for certain hardware scenarios, like Cougar + 3 axis rudders, or Cougar with homemade rudders. By default the CCP suggests that you save your calibration data in a file called calibration.ccf so for the moment let's stay with this default file.

The next checkbox is ticked by default and says "Make this the Active Calibration File." When the CCP wants to send calibration data to the Cougar, it acquires this data from the calibration file that you have selected as the Active Calibration File (ACF). So if you create several calibration files, you must always select one as the ACF.

The next checkbox is also ticked by default and quite simply, ensures that your Cougar is placed into manual calibration mode when you press the Save button. It also downloads the axis settings currently displayed in the CCP's tabs.

So press the Save button, and the CCP writes that calibration data to calibration.ccf or whatever filename you've chosen, assigns that as the Active Calibration File if you didn't deselect that option, and puts the Cougar into Manual Calibration mode. You'll see a "Settings have been applied to the device" Window ….

And when you press the “OK” button, you’ll be returned to the CCP:

You can see now that the Calibration mode has been changed to Manual calibration, and that the Axis Response has also changed to User mode. You cannot select the Default mode axis response when in manual calibration mode (after all, default mode is with auto calibration) so it is greyed out. The Manual calibration mode is no longer disabled and is actually selected, showing that we're in manual calibration mode. And now we see that calibration.ccf has been selected as the active calibration file, the file that supplies your calibration data for manual calibration mode.

Hope that's all clear to you now ;)

The contents of a Cougar Calibration File

Now, the whole aim of that calibration routine is to generate calibration data that can be saved in a Cougar Calibration File (*.ccf). These files are stored in your HOTAS\Calib folder. But what do they contain?

Well here’s what my calibration.ini file contains (I’ve added the axis names) after calibrating my Cougar with no rudders attached:

[Calibration]

0=41148,32256,23876 	Joystick X axis

1=27772,35294,45084	Joystick Y axis

2=43968,29824,18240	Microstick X

3=46848,35968,21184	Microstick Y

4=5600,14336,23072		Throttle

5=0,28672,57120		Antenna

6=0,28224,57120		Range

7=0,0,0			Rudder

8=0,0,0			These last two

9=0,0,0			are toebrake axes

AxesStatus=2 			For internal use – relates to hardware attached

Apart from the initial [Calibration] line and the final AxesStatus line, there are 10 lines, one for each axis. The format for each line is:

Axis number = minimum, centre, maximum positions

It doesn’t matter by the way that for some axes the values seem to increase, and for others, decrease as you read from left to right. So that’s all that the file contains. So what?

The significance of the calibration data in the active calibration file is that for manual calibration mode, whenever you press the Apply button in the CCP, or download a profile (*.tmc), calibration data is read in from the active calibration file and downloaded to the manual calibration memory in the Cougar.

Changes to the relationship between Profiles and Calibration data

This section really only applies to people who've owned a Cougar before this version of the software came out that moved towards using ccf files for calibration data. Previously, profiles (*.tmc files) contained axis settings and calibration data, and one of the reasons why the first version of this document came out, was to explain how to use profiles properly with manual calibration, and how to work around the bugs in the software. These bugs are all fixed and the contents of profiles have changed. Here's a very important point to take on board then:

Profiles contain axes settings only

That is they contain the following information for each axis: Curve, Reverse, Swap, Disable, DeadZones. They no longer contain any calibration information. So you can quite happily send a profile to a friend or download one from say Cougar World, and apply the axes settings within that profile without affecting your calibration.

One other change compared to the previous software: When you update your firmware, the calibration files are not deleted. You therefore do not need to write protect them anymore.

Summary and final details

- There are two forms of calibration for the Cougar: Auto and manual calibration.

- Auto calibration mode relies on you moving your axes to maximum and minimum positions each time you power up your Cougar. You can use the CCP or programming statements to change axis settings, and this is independent of your auto calibration data.

- Manual calibration relies on you running through the manual calibration routine, saving that information in a Cougar Calibration File, and assigning a calibration file to be the Active Calibration File.

- You cannot use manual calibration mode if you do not have calibration data saved and an active calibration file selected, that applies to your hardware setup.

- Profiles contain axes settings only.